一、Display panel and key definition

menu        UP      DOWN       ENTER

Menu key: Select function

Up key: increase the parameters step by step

Down key: parameter decrement

Confirm key: confirm and save

二、Menu function

Press the menu key after power on, and the menu menu menu will appear in turn; Press the up or down key to modify the function parameters, and press the OK key to save the current function and parameters (with power down memory after saving).

Menu menu：

| A001 |  | A512 | Set the address code, modify the address code (A001 ~ A512) up or down, and click OK to save. |
| --- | --- | --- | --- |
| CH10 |  | CH73 | Switch up or down the three channels CH04, ch10 and ch73, and click OK to save. Default ch10 |
| M000 |  | M127 | Effect mode, switch the built-in effect up or down, and click OK to save. |
| S000 |  | S255 | Modify the operation speed of hexagonal built-in effect upward or downward (s000 ~ s255), and press the OK key to save. |
| Soud |  | Soud | Voice mode. |
| R255 |  | R000 | R red light bulb dimming, modify the brightness of red light bulb (r000 ~ r255) up or down, and click OK to save. |
| G255 |  | G000 | G green light bulb dimming, modify the brightness of green light bulb (G000 ~ g255) up or down, and click OK to save. |
| B255 |  | B000 | B blue light bulb dimming, modify the brightness of blue light bulb (b000 ~ B255) up or down, and click OK to save. |
| W255 |  | W000 | W monochrome lamp brightness, modify the monochrome lamp brightness up or down (w000 ~ W255), and press the OK key to save. |
| T000 |  |  | Display the temperature. For example, t045 indicates that the current lamp temperature is 45 ℃ |

三、Master slave control

Two or more identical lamps are connected with DMX three core signal wires. All lamps are set to any address code from A001 to A512, and any one is set as the host. Other lamps are slave, and all slave displays do not flash; When the master is used to adjust the effects of gradient, pulse change, jump change, voice control and self walking, all slave machines will synchronize the effects of gradient, pulse change, jump change, voice control and self walking.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash out of sync.

2. All lamps must be turned off when the DMX512 console is turned off before the master and slave can work.

四、Factory settings

In case of any address code from A001 to A512, press the menu key for 3 seconds to enter the factory setting. Factory settings are mainly the functions of lamp output power, fan setting mode, setting temperature protection point and sending parameters. The factory sets any mode and exits by pressing the menu key for 3 seconds.

Factory setting table：

| R255 |  | R020 | R red lamp bead current setting, modify the hexagonal red lamp bead current (r020-r255) up or down, and click OK to save. |
| --- | --- | --- | --- |
| G255 |  | G020 | G green light bulb current setting, modify the hexagonal green light bulb current (g020-g255) up or down, and click OK to save. |
| B255 |  | B020 | B blue light bulb current setting, modify the hexagonal blue light bulb current (b020-b255) up or down, and press the OK key to save. |
| W255 |  | W020 | W monochrome lamp current setting, modify the monochrome lamp current setting up or down (w020 ~ W255), and click OK to save. |
| FAN1 |  | FAN0 | Fan setting: start the fan when the fan0 light is on, start the fan when Fan1 reaches the set temperature protection point, and click OK to save. |
| T040 |  | T-105 | Set the temperature protection point, modify the parameters up or down (40 ℃ ~ 75 ℃), and press the OK key to save. |
| Send |  | Send | Send the factory setting parameters of the machine up or down to all other lamps connected in parallel with three core signal wires; Confirm the sending parameters and press the menu key for 3 seconds |

五、DMX512Console

After power on, the address codes of all lamps are set. Connect all lamps with three core signal wires in parallel to DMX512 console, and the address code will stop flashing, indicating that the signal of DMX512 console has been sent to lamps. Use DMX512 console to control relevant functions according to the instructions of each channel.

CH10Channel description：

| passageway | Channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | Total open light |
| 2 | 000-255 | Stroboscopic |
| 3 | 000-255 | Red light bead linear dimming |
| 4 | 000-255 | Green light bead linear dimming |
| 5 | 000-255 | Blue light bead linear dimming |
| 6 | 000-255 | White light bead linear dimming |
| 7 | 000-255 | RGB mode self-propelled |
| 8 | 000-255 | Mode speed |
| 9 | 000-255 | Background color |
| 10 | 000-255 | Background color brightness |

CH4Channel description：

| passageway | Channel value | basic function |
| --- | --- | --- |
| 1 | 000-255 | Red light bead linear dimming |
| 2 | 000-255 | Green light bead linear dimming |
| 3 | 000-255 | Blue light bead linear dimming |
| 4 | 000-255 | White light bead linear dimming |

CH73Channel description：

| Channel value | Mode code | effect |
| --- | --- | --- |
| 1 | 000-255 | Group 1 Linear dimming of red light beads |
| 2 | 000-255 | Group 1 green light bead linear dimming |
| 3 | 000-255 | Group 1 Linear dimming of blue light beads |
| 4 | 000-255 | Group 2 linear dimming of red light beads |
| 5 | 000-255 | Group 2 green light bead linear dimming |
| 6 | 000-255 | Group 2 linear dimming of blue light beads |
| … | … | … … |
| 70 | 000-255 | Group 24 linear dimming of red light beads |
| 71 | 000-255 | Group 24 green light bead linear dimming |
| 72 | 000-255 | Group 24 linear dimming of blue light beads |
| 73 | 000-255 | White light bead linear dimming |

六、Mode effect

| Channel value | Mode code | effect |
| --- | --- | --- |
| 0-1 | 0 | No effect |
| 2-3 | 1 | Effect one |
| 4-5 | 2 | Effect II |
| 6-7 | 3 | Effect three |
| 8-9 | 4 | Effect 4 |
| … | … | … … |
| 250-251 | 125 | Effect 125 |
| 252-253 | 126 | Effect 126 |
| 254-255 | 127 | Effect 127 |

7、 Technical parameters:

Voltage: AC100 ~ 220V 50 / 60Hz

Power: 72 * 0.25W (18W) + 60W

Lamp beads: 72 5050 three color LED lamp beads + long white light lamp beads

Control mode: DMX512, self-propelled, master-slave, voice control, with RDM function.

Channel: CH010, CH04, ch73

Dimming: 32bit 0 ~ 100% linear dimming

Features: dyeing + Flash + auxiliary light

Operating temperature: - 30 ℃ ~ 50 ℃

Stroboscopic frequency: 1 ~ 30Hz

Appearance: metal, black

Connection mode: DMX512 input / output / power input / output.

IP class: IP20

Size: weight: