RH-COB07A  7-eye audience light DMX channel function table

AH70 DH76

11 channel mode (A001 status)

| channel | function | explain |
| --- | --- | --- |
| CH1 | Total dimming | Total dimming, from dark to bright |
| CH2 | True White 1 Dimming | Positive white 1 dimming, from dark to bright |
| CH3 | True White 2 Dimming | Positive white 2 dimming, from dark to bright |
| CH4 | True White 3 Dimming | Positive white 3 dimming, from dark to bright |
| CH5 | True White 4 Dimming | True white 4 dimming, from dark to bright |
| CH6 | Warm White 1 Dimming | Warm white 1 dimming, from dark to bright |
| CH7 | Warm White 2 Dimming | Warm white 2 dimming, from dark to bright |
| CH8 | Warm White 3 Dimming | Warm white 3 dimming, from dark to bright |
| CH9 | Total stroboscopic | Global strobe, slow to fast |
| CH10 | Function selection | 0-50: Dimming mode; |
|  |  | 51 － 100: color selection; |
|  |  | 101-150: Jump to self walk effect; |
|  |  | 151-200: Gradient effect; |
|  |  | 201-249 pulse change effect; |
|  |  | 250-255: Voice control (CH11 selects voice control 1 and 2) |
| CH11 | Functional speed | Function speed, from slow to fast (select voice strobe and jump during voice control) |

6-channel mode (d001)

| channel | function | explain |
| --- | --- | --- |
| CH1 | Total dimming | Total dimming, from dark to bright |
| CH2 | True White Dimming | Positive white dimming, from dark to bright |
| CH3 | Warm White Dimming | Warm white dimming, from dark to bright |
| CH4 | Total stroboscopic | Global strobe, slow to fast |
| CH5 | Function selection | 0-50: Dimming mode; |
|  |  | 51 － 100: color selection; |
|  |  | 101-150: Jump to self walk effect; |
|  |  | 151-200: Gradient effect; |
|  |  | 201-249 pulse change effect; |
|  |  | 250-255: Voice control (CH6 selects voice control 1 and 2) |
| CH6 | Functional speed | Function speed, from slow to fast (select voice strobe and jump during voice control) |

2、 Description of nixie tube display menu

| order number | content | Value range | Function and Description |
| --- | --- | --- | --- |
| 1 | A001 | 001- 512 | DMX512 address code setting - 11 channel mode |
| 2 | d001 | 001- 512 | DMX512 Address Code Settings - 6 Channel Mode |
| 3 | r255 | 000 - 255 | Positive white 1 dims from dark to bright. |
| 4 | G255 | 000 - 255 | Positive white 2 dims from dark to bright. |
| 5 | b255 | 000 - 255 | Positive white 3 dims from dark to bright. |
| 6 | U255 | 000 - 255 | True white 4 dimming, from dark to bright |
| 7 | y255 | 000 - 255 | Warm White 1 dims from dark to bright. |
| 8 | P255 | 000 - 255 | Warm White 2 dims from dark to bright. |
| 9 | n255 | 000 - 255 | Warm White 3 dims from dark to bright. |
| 10 | FH00 | 00 - 99 | No strobe at 00, global strobe at 01-99, with a slow to fast speed. |
| 11 | CL01 | 01 - 49 | Manual color selection mode. |
| 12 | CC99 | 01 - 99 | Jump from slow to fast. |
| 13 | DE99 | 01 - 99 | Gradient, from slow to fast. |
| 14 | CP99 | 01 - 99 | Pulse changes from slow to fast. |
| 15 | SU01 | 01 - 02 | Mode: 1 is strobe jump, 2 is jump |

- Master Slave Description

Automatically receive slave signals in the d001 or A001 state, without requiring additional settings.