Full color animation laser manual

1. play mode selection: short press MUN key to switch: built - in effect mode, professional DMX mode

Type, simple DMX mode

1 - 1: built - in effect mode: short press enter key, you can switch the built - in self - walking mode, built - in voice control mode.

1-1-1: built-in self-walking mode display: se + number, fourth decimal point out. ( number is the serial number being played )

1 - 1 - 2: built - in voice mode display: se + number, fourth decimal point on. ( number is the serial number being played )

1 - 1 - 3: press add - subtract key to switch serial number.

1 - 2: professional DMX mode display: A + number. ( number for address code value ) ( 32 channels )

1 - 2 - 1: when there is DMX signal, the fourth decimal point is bright, when there is no DMX signal, the fourth decimal point is out,

1 - 3: simple DMX mode display: E + number. ( number for address code value ) ( 6 channels )

1 - 3 - 1: when there is a DMX signal, the fourth decimal point is lit, and when there is no DMX signal, the fourth decimal point is extinguished.

1.2: channel menu for DMX mode:

CH1 > switch lamp:

[ 0, closed light ] [ 1-255, turn on the light ]

CH2 > action sound control self - walking control:

[ 0-31, self-walking, built-in field most ] [ 32-63, self-walking, built-in sequence ] [ 64-95, self-walking, built-in mixing ] 96-127, retention ]

[ 128-159, sound control, built-in scene ] [ 160-191, voice control, built-in sequence ] [ 192-223, voice control, built-in mixing and kneading] [224-255, BaO

Stay].

CH3 > effect library ( library ) select:

[0 – 15 library no1] [16 – 31 library no2]……[192 – 223 library no15] [224 – 255 library no0]

Note 1: when the library number exceeds the number of this lamp, the system will be replaced by the maximum library number.

Note2: this channel has no function when ch2 is built - in mixed broadcast.

CH4 > scene selection:

( 0 - 255, each value corresponds to a pattern, the value exceeds the number of patterns, the system replaced with the maximum figure number ).

Note1: this channel does not function when ch2 is a built - in mixed broadcast or built - in sequence.

CH5 > color selection:

[ 0-31, color ] [ 32-63, red ] [ 64-95, yellow ] [ 96-127, green ] [ 128-159, indigo ] [ 160-191, blue )[ 191-223, purple ] [ 224-255, white ]

CH 6 > speed adjustment:

[ 0, original speed ] [ 1-255, speed adjustment ( slow - fast ) ]