HAZE MACHINE

MANUAL

Please read this manual carefully before using

Thank you for having chosen our company's haze machine. For your own safety, please read this user manual carefully before installing the device.

Attention

To keep your machine in good working order please remember to:

●Use only water-based, non-toxic haze liquid.

●Keep the machine clean.

●Turn the power off before adding liquid. Turn the power off when not in use.

Also..

●Always make sure that you are connecting to the proper voltage , and that

the line voltage you are connecting to is not higher than that stated on the decal or rear panel of the fixture.

This product is intended for indoor use only.

To prevent risk of fire or shock , do not expose this unit to rain or moisture.

●Make sure there are not flammable material close to the unit while operating.

●The unit must be installed in a location with adequate ventilation, at least20in (50cm) from adjacent surfaces. Be sure that no ventilation slots re blocked.

●Always disconnect from the power source before servicing or replacing fuse and be sure to replace with same fuse size and type.

●In the event of a serious operating problem. Stop using the unit immediately. Never try to repair the unit by yourself. Repairs carried out by unskilled people can lead to damage or malfunction.Please contact the nearest authorized technical assistance center. Always use the same type spare parts.

●Never connect the device to a dimmer pack.

●Make sure the power cord is never crimped or damaged.

●Never disconnect the power cord by pulling or tugging on the cord.

Routine Maintenance and common faults

Periodically clean your machine to reduce replacement parts and repair expenses.

1. The poor fog liquid can cause blockage, it is recommended to use high quality fog liquid.

2. The machine might be blocked due to high concentration fluid, not clean fluid or overheat, so regular care andmaintenance can ensure long-term use.After about 40 hours continually use, please use 80% distilled water and 20% white vinegar to remove the accumulated impurities inside the heat pipe , as following:

1)washing the pipe in the tank and the filter head.

2) Pour out the fog liquid , replacing the cleaning liquid (80% distilled water and 20% white vinegar), turn on the power , keep it operating till the cleaning liquid runs out.

3. If the fog turns less, and the pump makes noise, or can not spray fog, you should turn off the power immediately and at the same time check the fluid tank, fuse and power plug. If all of these are normal, please connect the power with the plug and try again, if the problems still can not be solved, you should contact supplier.

Main Features

●DMX controllable water-based haze machine.

●Haze output and blower speed adjustment. Timer with interval and duration settings.6 button control panel with LCD display.

●Optional low liquid shutdown.

●5-pin DMX connectivity.

●6-position moveable haze guide for directional output control.

●2-channel DMX control ( fan speed , output volume).

Technical parameters:

Power consumption:1200W

Voltage:AC220V-240V  50Hz-60Hz

Warm-up time:150 seconds

Tank capacity:3.0L

Liquid consumption : 60min/L(1L)

DMX channels : 2 channel

5 pin DMX input /output

Weight:11kg

Size:45.5*40.5*22.5cm

Setting Up

Step 1 : place the machine on a flat surface and in suitable large area with at least 50cm open space around the machine .

Step 2 : fill the liquid tank with quality water-based ,non-toxic haze liquid.

Step 3 : connect the machine to suitable rated power supply.

Step 4 : the LCD menu will now display “warming up " in preparation for use . Warming can take up to 150 seconds.

Step 5: when it is ready , the LCD menu will display “Ready Haze", and at this point, you could manually start making haze by holding down the MANUAL button on the control panel.

Step 6 : turn off the machine, unplug the unit from its power source.

Note : Before replacing a fuse , disconnect power cord. Always replace with the same type and rating of fuse.

Setting up DMX control

Step 1: connect the male connector of the DMX cable to the female connector (output) on the controller.

Step 2: connect the female connector of the DMX cable to the first unit's male connector (input).

Note : it doesn't matter which unit address is the first one connected. We recommend connecting them in terms of their proximity to the controller, rather than connecting the lowest fixture number first , and so on.

Step 3: connect others in the chain from output to input as above . Place a

DMX terminator on the output of the final unit to ensure best communication.

DMX Reference ( 2-channle mode)

Channel 1: control volume output.

Channel 2: control fan speed.

LCD DISPLAY

Ready

Haze

F

T

M

V

| Button | Function |
| --- | --- |
| FUNCTOIN | Enters the programming mode/ Long press to enter background mode |
| UP | Value adjustment upward |
| DOWN | Value adjustment downward |
| TIMER | Starts the timer controlled operation |
| VOLUME | Starts or stops all operation |
| MANUAL | Exit/Press and hold to display temperature |

●To navigate the LCD control menu options, use the FUNction button to scroll through the various menu options Then to scroll through a menu options selectable values, use the UP/DOWN buttons. Press FUNCTION button to save any changes made and return to navigation the menu.

●Note : once a DMX signal is received,manual mode is deactivated and the buttons TIMER,START/STOP and MANUSL are locked.

Control Panel FUNCTION Structure

| Dmx_Addr | 001-512 | Set the starting DMX address |
| --- | --- | --- |
| Fog Vol | 1%-100% | Set the volume output |
| Fan Vol | 30-100 | Set the fan speed |
| Time | 1-1500(seconds) | Duration time of timed operation |
| Interval | 1-1500(seconds) | Pause time between timed output |

>>>

〉〉〉〉〉: means the machine is heating.

Ready  : means the machine can emit the haze .

Dmx- Addr 1-512 : press the button MENU, it will show DMX512 ADD001-512, you Can set the desired DMX address by the buttons UP or DOWN.

Fog Vol 1%-100% : press the button FUNCTION, till it shows Volume set 1-100% , you can set the volume from 1%-100% by the Button UP or DOWN.

Fan Vol：30-100 : press the button FUNCTION, till it showsFan Vol 30-100 ,you can set the fan speed from 30 to 100 by the Button UP or DOWN.

Time 1-1500s : press the button FUNCTION,till it shows Duration set 1-1500s, you can set the continuous haze output from 1-1500 seconds by

the buttons UP or DOWN.

Interval 1-1500s : press the button FUNCTION, till it shows Interval set 1s-1500s, you can set the interval haze emission timer from 1-1500 seconds by the Button UP or DOWN.

TIMER : when the interval time and duration time set, press the button TIMER, the machine starts to work, when press again it stops working.

VOLUME : when the haze ready to emit , press the button VOLUME, the haze will emit continuously. Press again , Enter the fan interface ,Click on Manual to exit work

Manual : Exit/Press and hold to display temperature