FULL COLER LAMAGING LAMP

Technical parameters

Voltage：AC90~240V Wide voltage range input

Channel：2/6CH

Power supply：300W

Operation:DMX512, Auto, Sound, Master-slave

Key Operation Instructions

1.Key Diagram：

MENU：Function UP：Add   DOWN：Dec   ENTER：Confirm

- Key Description

MENU Function:Press the function key MENU to cycle through the functions, effects and other parameters of table (a); press UP or DOWN to modify their parameter values; press ENTER to confirm.

UP Add:Press the key to achieve plus, and long press for more than 2 seconds to continuously add.

DOWN Dec:Press the key to achieve minus, and long press for more 	than 2 seconds to continuously minus.

ENTER Confirm:Press this key to save the currently set parameter function and run the saved function when the power is turned on.

DMXDISPLAY

| No | Show | Function | Instruction |
| --- | --- | --- | --- |
| 1 | ADDR-2CH | 001-512 | DMX-512 address set,2 Channels mode |
| 2 | ADDR-6CH | 001-512 | DMX-512 address set,6 Channels mode |
| 3 | WARM | 000-255 | UP、DOWN Change brightness |
| 4 | COLD | 000-255 | UP、DOWN Change brightness |
| 5 | STROBE | 01-99 | UP、DOWN Change the speed 。 |
| 6 | JUMP | 01-99 | UP、DOWN Change the speed |
| 7 | GRADIENT | 01-99 | UP、DOWN Change the speed |
| 8 | PUSLE | 01-99 | UP、DOWN Change the speed |
| 9 | SOUND | 1 - 9 | UP、DOWN Change the effects |

DMX512Control:2Channel Model(ADDR-2CH)

| CH | Display | Function |
| --- | --- | --- |
| CH1 | U Dimmer | Dimmer- Dark to Bright |
| CH2 | C Dimmer | Dimmer- Dark to Bright |

DMX512Control:6Channel Model(ADDR-6CH)

| CH | Display | Function |
| --- | --- | --- |
| CH1 | Dimmer | U、C dimmer : from dark to bright |
| CH2 | U Dimmer | Dimmer- Dark to Bright |
| CH3 | C Dimmer | Dimmer- Dark to Bright |
| CH4 | Strobe | U、C strobe: from slow to fast |
| CH5 | Function choice | 0~50: DMX 8CH Control. 51~100: Colors Jump Change. 101~150: Colors Gradate. 151~200: Colors Pulse Change. 201~255: Sound-Active |
| CH6 | Function speed | Speed function, from slow to fast |